SIMATS ENGINEERING

ASSESSMENT TOOL 1

STUDENT NAME: G.Padma Bhindya

REGISTER NO: 192565049

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Description about the assessment tool:

Short-answer questions effectively measure conceptual understanding, logical reasoning, and the ability to apply theoretical knowledge without requiring lengthy descriptive responses. This assessment method is also efficient for evaluating multiple OSI-related concepts within a limited time while ensuring objective and consistent evaluation.

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25% | Demonstrates a thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

ANSWERS

1. File Transfer using Transport and Data Link Layer

Given

- File size = 150 MB

- Bandwidth = 50 Mbps

- Segment size = 1500 bytes

- Data Link Layer overhead = 40 bytes/frame

Assume 1 MB = 1,000,000 bytes

Step 1: Convert File Size

150 MB=150×1,000,000=150,000,000 bytes150 \text{ MB} = 150 \times 1,000,000 = 150,000,000 \text{ bytes}150 MB=150×1,000,000=150,000,000 bytes

Step 2: Calculate Number of Segments

Segments=150,000,0001500=100,000\text{Segments}=\frac{150,000,000}{1500}=100,000Segments=1500150,000,000​=100,000

Each segment becomes one frame.

Number of Frames = 100,000

Step 3: Calculate Total Data Link Layer Overhead

Overhead per frame = 40 bytes

100,000×40=4,000,000 bytes100,000 \times 40 = 4,000,000 \text{ bytes}100,000×40=4,000,000 bytes

= 4 MB

Step 4: Total Data Transmitted

150,000,000+4,000,000=154,000,000 bytes150,000,000+4,000,000=154,000,000 \text{ bytes}150,000,000+4,000,000=154,000,000 bytes

Convert into bits:

154,000,000×8=1,232,000,000 bits154,000,000 \times 8=1,232,000,000 \text{ bits}154,000,000×8=1,232,000,000 bits

Step 5: Transmission Time

Bandwidth

50 Mbps=50,000,000 bits/sec50\text{ Mbps}=50,000,000\text{ bits/sec}50 Mbps=50,000,000 bits/sec 1,232,000,00050,000,000=24.64 seconds\frac{1,232,000,000}{50,000,000}=24.64\text{ seconds}50,000,0001,232,000,000​=24.64 seconds

Final Answer

- Number of Segments = 100,000

- Number of Frames = 100,000

- Total Data Link Layer Overhead = 4,000,000 bytes (4 MB)

- Total Transmission Time = 24.64 seconds

Explanation

The Transport Layer divides large files into smaller segments and ensures reliable communication using sequence numbers, acknowledgements, retransmissions, and error recovery.

The Data Link Layer encapsulates each segment into a frame by adding headers and trailers. It performs error detection using CRC, manages frame synchronization, and provides reliable node-to-node communication.

2. Sliding Window Protocol

Given

- Window size = 6 frames

- Frame size = 1024 bytes

- Total frames = 48

Step 1: Number of Windows

486=8\frac{48}{6}=8648​=8

Windows required = 8

Step 2: Retransmissions

One frame is lost in every window.

8×1=88 \times 1=88×1=8

Retransmissions = 8

Final Answer

- Number of Windows = 8

- Retransmissions = 8 frames

Flow Control Explanation

Flow control prevents the sender from transmitting data faster than the receiver can process it. Sliding Window Protocol improves network efficiency by allowing multiple frames to be transmitted before receiving acknowledgements, thereby reducing waiting time and increasing throughput.

3. IP Fragmentation

Given

- IP Packet = 12,000 bytes

- MTU = 1500 bytes

- IP Header = 20 bytes

Step 1: Payload per Fragment

1500−20=1480 bytes1500-20=1480\text{ bytes}1500−20=1480 bytes

Step 2: Number of Fragments

120001480=8.11\frac{12000}{1480}=8.11148012000​=8.11

Therefore,

Fragments = 9

Step 3: Header Overhead

Each fragment requires a new header.

9×20=180 bytes9 \times20=180\text{ bytes}9×20=180 bytes

Final Answer

- Number of Fragments = 9

- Header Overhead = 180 bytes

Explanation

The Network Layer fragments packets whenever they exceed the MTU of a network. Each fragment contains its own IP header with fragment offset and identification fields. At the destination, all fragments are reassembled to reconstruct the original packet, ensuring successful delivery.

4. Sliding Window Protocol

Given

- Window size = 6

- Total Frames = 48

Windows

48/6=848/6=848/6=8

Retransmissions

One frame lost in every window

8×1=88 \times1=88×1=8

Final Answer

- Transmission Windows = 8

- Retransmissions = 8

Flow Control

Flow control ensures the sender does not overload the receiver. It balances transmission speed, minimizes congestion, reduces packet loss, and improves communication efficiency.

5. Session Layer Synchronization

Given

- Video File = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

Step 1: Number of Checkpoints Before Failure

Checkpoints occur at

60

120

180

240

300

360

Total = 6 checkpoints

Step 2: Retransmission

Last checkpoint = 360 MB

Failure = 390 MB

Retransmission

390−360=30 MB390-360=30\text{ MB}390−360=30 MB

Final Answer

- Checkpoints Created = 6

- Data Retransmitted = 30 MB

Importance of Synchronization

The Session Layer inserts synchronization points (checkpoints) during long transmissions. If communication fails, data transfer resumes from the last checkpoint instead of restarting from the beginning, saving time and bandwidth.

6. Presentation Layer Compression and Encryption

Given

Original File = 900 MB

Compression = 40%

Encryption Overhead = 5%

Step 1: Compressed Size

Remaining

900×0.60=540 MB900\times0.60=540\text{ MB}900×0.60=540 MB

Step 2: Encrypted Size

540×1.05=567 MB540\times1.05=567\text{ MB}540×1.05=567 MB

Step 3: Overall Reduction

Reduction

900−567=333 MB900-567=333\text{ MB}900−567=333 MB

Percentage

333900×100=37%\frac{333}{900}\times100=37\%900333​×100=37%

Final Answer

- Compressed File = 540 MB

- Encrypted File = 567 MB

- Total Reduction = 37%

Explanation

The Presentation Layer compresses data to reduce transmission size and encrypts it to ensure confidentiality. Compression improves transmission speed, while encryption protects sensitive information from unauthorized access.

7. FTP Application

Given

Data = 3 GB

Throughput = 120 Mbps

Each Request = 3 MB

Step 1: Convert Data

3 GB=3000 MB3\text{ GB}=3000\text{ MB}3 GB=3000 MB

Step 2: Number of Requests

3000/3=10003000/3=10003000/3=1000

Step 3: Convert Data to Bits

3×8=24 Gb3\times8=24\text{ Gb}3×8=24 Gb

Step 4: Transmission Time

24000/120=200 seconds24000/120=200\text{ seconds}24000/120=200 seconds

Final Answer

- Number of Requests = 1000

- Transmission Time = 200 seconds

Explanation

The Application Layer provides user services such as FTP, HTTP, and SMTP. It ensures reliable file transfer through authentication, file management, error reporting, and communication with lower layers.

8. End-to-End Delay

Given

Application = 4 ms

Transport = 3 ms

Network = 5 ms

Data Link = 2 ms

Physical = 1 ms

Propagation = 18 ms

Transmission = 12 ms

Calculation

Layer Processing

4+3+5+2+1=15 ms4+3+5+2+1=15\text{ ms}4+3+5+2+1=15 ms

Total Delay

15+18+12=45 ms15+18+12=45\text{ ms}15+18+12=45 ms

Final Answer

Total End-to-End Delay = 45 ms

Explanation

- Application Layer prepares user data.

- Transport Layer performs segmentation and reliability.

- Network Layer handles routing.

- Data Link Layer frames the data.

- Physical Layer transmits bits.

- Propagation delay is the time for signals to travel through the medium.

- Transmission delay is the time required to place all bits onto the communication channel.

9. Protocol Overhead

Given

Useful Data = 80 MB

Frame Size = 1600 bytes

Protocol Overhead = 80 bytes

Step 1: Useful Data per Frame

1600−80=1520 bytes1600-80=1520\text{ bytes}1600−80=1520 bytes

Step 2: Total Frames

Useful Data

80,000,000 bytes80,000,000\text{ bytes}80,000,000 bytes 80,000,0001520=52,632 frames\frac{80,000,000}{1520}=52,632\text{ frames}152080,000,000​=52,632 frames

Step 3: Total Overhead

52,632×80=4,210,560 bytes52,632\times80=4,210,560\text{ bytes}52,632×80=4,210,560 bytes

≈ 4.21 MB

Step 4: Transmission Efficiency

15201600×100=95%\frac{1520}{1600}\times100=95\%16001520​×100=95%

Final Answer

- Frames Required = 52,632

- Total Overhead = 4,210,560 bytes (≈4.21 MB)

- Transmission Efficiency = 95%

Explanation

Protocol overhead consists of headers and trailers added by different OSI layers. Although overhead slightly reduces efficiency, it is essential for addressing, routing, synchronization, error detection, and reliable communication.

10. Healthcare Network Communication

Given

Medical Record = 240 MB

Compression = 30%

Segment Size = 1400 bytes

Frame Overhead = 60 bytes

Step 1: Compressed Size

240×0.70=168 MB240\times0.70=168\text{ MB}240×0.70=168 MB

Compressed Data

168,000,000 bytes168,000,000\text{ bytes}168,000,000 bytes

Step 2: Number of Segments

168,000,0001400=120,000\frac{168,000,000}{1400}=120,0001400168,000,000​=120,000

Segments = 120,000

Step 3: Total Overhead

120,000×60=7,200,000 bytes120,000\times60=7,200,000\text{ bytes}120,000×60=7,200,000 bytes

= 7.2 MB

Step 4: Total Data over Physical Layer

168+7.2=175.2 MB168+7.2=175.2\text{ MB}168+7.2=175.2 MB

Final Answer

- Compressed File Size = 168 MB

- Number of Segments = 120,000

- Protocol Overhead = 7,200,000 bytes (7.2 MB)

- Final Data Transmitted = 175.2 MB

Explanation

Multiple OSI layers cooperate to ensure secure and reliable communication:

- Presentation Layer: Compresses and encrypts the medical data.

- Transport Layer: Segments the data and provides reliable delivery using acknowledgements and retransmissions.

- Network Layer: Selects the best route to the destination.

- Data Link Layer: Adds frame headers and trailers for error detection and reliable node-to-node communication.

- Physical Layer: Transmits the data as electrical, optical, or wireless signals through the communication medium.