SIMATS ENGINEERING

ASSESSMENT TOOL 2

STUDENT NAME: G.Padma Bhindya

REGISTER NO: 192565049

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 30%

Annexure A

CONCEPT MAPPING

1. A partially completed concept map is given below:

Application → Presentation → ______ → Transport → Network → ______ → Physical.

Analyze the missing layers and explain how their functions are essential for reliable communication.

2. Construct a concept map linking data units (Segment, Packet, Frame, Bits) with their respective OSI layers and analyze how transformation of data units supports transmission.

3.Given the concept map:

DataLink→MACAddress→LocalDelivery
Network → IP Address → Global Delivery

Explain how these relationships ensure complete data delivery across networks.

4. A concept map shows:

ErrorControl
→DataLinkLayer→ErrorDetection
→ Transport Layer → Error Correction

Analyze how the interaction between these layers improves reliability in data transmission.

5. Develop a concept map for web browsing using OSI layers and analyze how failure in one layer affects the entire communication process.

6. A network issue is represented in the concept map:

Physical ✔ → Data Link ✔ → Network ✖ → Transport ✖ → Application ✖

Analyze the problem and explain how the concept map helps in troubleshooting.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Concept Identification | 20% | Accurately identifies all relevant OSI layers, concepts, and relationships | Identifies most concepts with minor omissions | Identifies limited concepts; some inaccuracies | Unable to identify key concepts correctly |
| Concept Mapping Structure | 25% | Constructs a clear, logical, and well-organized concept map with proper hierarchy and connections | Concept map is mostly clear with minor structural issues | Concept map lacks clarity, organization, or proper flow | No clear structure or incorrect mapping |
| Linking & Relationships | 20% | Clearly establishes correct relationships between layers, functions, and processes | Relationships are mostly correct with minor errors | Limited or partially correct relationships | Incorrect or missing relationships |
| Application & Analysis | 20% | Effectively applies concepts to explain processes (e.g., encapsulation, error control) with strong reasoning | The application is correct but lacks depth in analysis | Basic application with weak explanation | Unable to apply concepts correctly |
| Explanation & Clarity | 15% | Provides a clear, concise, and well-structured explanation supporting the concept map | Explanation is understandable with minor gaps | Explanation is unclear or incomplete | No clear or meaningful explanation |

ANSWERS

1. Missing OSI Layers in the Concept Map

Given Concept Map

Application → Presentation → ______ → Transport → Network → ______ → Physical

Completed Concept Map

Application

↓

Presentation

↓

Session

↓

Transport

↓

Network

↓

Data Link

↓

Physical

Analysis

The missing layers are:

- Session Layer

- Data Link Layer

Functions of the Session Layer

- Establishes communication sessions between devices.

- Maintains and manages active sessions.

- Synchronizes data transfer using checkpoints.

- Restores communication after interruptions.

Functions of the Data Link Layer

- Converts packets into frames.

- Adds MAC addresses for local communication.

- Detects transmission errors using CRC (Cyclic Redundancy Check).

- Controls access to the communication medium.

- Ensures reliable node-to-node data transfer.

Conclusion

The Session Layer ensures uninterrupted communication by managing sessions and synchronization, while the Data Link Layer guarantees reliable frame transmission and error detection. Together, these layers contribute to secure and reliable communication throughout the OSI model.

2. Concept Map for Data Units and OSI Layers

Concept Map

Application

│

Presentation

│

Session

│

Transport

↓

Segment

│

Network

↓

Packet

│

Data Link

↓

Frame

│

Physical

↓

Bits

Analysis

As data moves down the OSI model, each layer transforms the data into a different Protocol Data Unit (PDU):

| OSI Layer | Data Unit |
| --- | --- |
| Transport Layer | Segment |
| Network Layer | Packet |
| Data Link Layer | Frame |
| Physical Layer | Bits |

- Transport Layer divides large data into segments.

- Network Layer adds logical addressing to create packets.

- Data Link Layer encapsulates packets into frames with MAC addresses.

- Physical Layer converts frames into bits for transmission through the communication medium.

Conclusion

The transformation of data units allows each OSI layer to perform its specific functions such as addressing, routing, framing, and physical transmission, ensuring efficient and reliable communication.

3. Concept Map: MAC Address and IP Address

Concept Map

Data Link Layer

│

↓

MAC Address

│

↓

Local Delivery

Network Layer

│

↓

IP Address

│

↓

Global Delivery

Analysis

The Data Link Layer uses MAC addresses, which are physical addresses assigned to network interfaces. MAC addresses are used for communication between devices on the same local network (LAN).

The Network Layer uses IP addresses, which are logical addresses that uniquely identify devices across different networks. Routers use IP addresses to forward packets from the source network to the destination network.

How They Work Together

- The IP address identifies the destination network.

- The MAC address identifies the destination device within that network.

- Routers use IP addresses for routing.

- Switches use MAC addresses for local delivery.

Conclusion

The combination of IP addressing and MAC addressing enables complete data delivery from the source device to the destination device across both local and global networks.

4. Concept Map: Error Control

Concept Map

Error Control

│

┌───────────────┴───────────────┐

│                               │

Data Link Layer               Transport Layer

│                               │

Error Detection              Error Correction

Analysis

The Data Link Layer performs error detection by adding error-checking information such as CRC (Cyclic Redundancy Check) to every frame. If an error is detected, the frame can be discarded or retransmitted.

The Transport Layer provides error correction by using sequence numbers, acknowledgements (ACKs), timers, and retransmissions. Protocols such as TCP ensure that lost or corrupted segments are resent until they are received correctly.

Interaction Between the Layers

- The Data Link Layer detects errors occurring between directly connected devices.

- The Transport Layer ensures end-to-end reliability across the entire network.

- Together, they minimize data loss and maintain communication accuracy.

Conclusion

The interaction between the Data Link Layer and the Transport Layer provides highly reliable communication by detecting transmission errors locally and correcting them across the complete communication path.

5. Concept Map for Web Browsing Using OSI Layers

Concept Map

User

│

Application Layer

(HTTP/HTTPS)

│

Presentation Layer

(Encryption - SSL/TLS)

│

Session Layer

(Session Management)

│

Transport Layer

(TCP)

│

Network Layer

(IP Routing)

│

Data Link Layer

(Frame Transmission)

│

Physical Layer

(Bits over Cable/Wi-Fi)

Analysis

During web browsing:

- Application Layer provides web services using HTTP/HTTPS.

- Presentation Layer encrypts and decrypts data using SSL/TLS.

- Session Layer establishes and maintains communication sessions.

- Transport Layer reliably transfers web pages using TCP.

- Network Layer routes packets across the Internet.

- Data Link Layer frames the packets and performs error detection.

- Physical Layer transmits bits through wired or wireless media.

Effect of Failure in One Layer

If any layer fails:

- Failure in the Application Layer prevents the browser from accessing websites.

- Failure in the Presentation Layer causes encryption problems, resulting in insecure or failed connections.

- Failure in the Session Layer disconnects active communication.

- Failure in the Transport Layer causes packet loss or incomplete page loading.

- Failure in the Network Layer prevents routing to the destination.

- Failure in the Data Link Layer results in frame errors.

- Failure in the Physical Layer completely disconnects the network.

Conclusion

Each OSI layer depends on the services provided by the lower layers. Failure in any layer interrupts the communication process and prevents successful web browsing.

6. Concept Map for Network Troubleshooting

Given Concept Map

Physical       ✔

↓

Data Link      ✔

↓

Network        ✖

↓

Transport      ✖

↓

Application    ✖

Analysis

The concept map indicates that:

- The Physical Layer is functioning correctly.

- The Data Link Layer is also functioning correctly.

- The failure starts at the Network Layer.

Since the Network Layer is responsible for logical addressing and routing, communication cannot proceed beyond this point.

Possible causes include:

- Incorrect IP address configuration.

- Invalid subnet mask.

- Incorrect default gateway.

- Router failure.

- Routing table errors.

- Network congestion.

Because the Network Layer fails:

- The Transport Layer cannot establish reliable connections.

- The Application Layer cannot access network services such as web browsing, email, or file transfer.

How the Concept Map Helps in Troubleshooting

The concept map helps identify the exact layer where communication fails.

Instead of checking every network component, the network administrator can focus directly on the Network Layer, reducing troubleshooting time and improving fault diagnosis.

Conclusion:

The concept map clearly shows that lower layers are functioning properly while the Network Layer is malfunctioning. Identifying the faulty layer allows faster troubleshooting, quicker repairs, and restoration of reliable communication.